- Monitors the wafer fabrication process by responding to in-line issues and dispositions of production wafers

- Qualifies wafer fabrication processes on new tools.

- Coordinates the scheduling of process qualifications and equipment down time with manufacturing or equipment engineering.

- Uses software to analyze data, identify trends to make decisions about processing product.

- Makes adjustments when process parameters are exceeded and monitors process parameters when tools return from maintenance.

- Executes in line changes (CPM) to the process or equipment.

- Communicates technical information within the department through presentations.

- Continuously revises Standard Operating Procedures (SOPs), SPEC, AM,

- Develops improved processes to increase capability.

Process development and documents- spec edit example:

PM SPEC updates for power cycle:

- Please exclude steps to temp down Ni Bath

- Please exclude step to run conditioning wafers on Ni bath

- For Ni bath, we should always debubble first before running any TFR wafers. Ni bath evaporates at higher rate as compared to Cu and Solder Bath.

- For Cu and Solder bath, please run TFR on 1 duet through each chemistry (Cu and SnAg) and make call for debubble.

If TFR passes then run TFR on remaining duets, if TFR fails then proceed with debubble on all duets followed by TFR

- Please update PM SPEC Tool Power Down procedure: Step 15-19 with procedure from BKM SPEC - section 6.5.11 - Step 4

Process development and documents: WP on process changes : 1) redefining the spec limits after increased amunt of product specific Flags. Changing the

Developmet of html based app for monitoring tool down time and tool

SLF meeting weekly and present the most important incidents of the week with all  troubleshooting process, and learnings . or ongoing incidents and then come up with the ARs and assign

Process troubleshooting, data analysis, collaboration, and implementing improvements.

⭐ Question:

"Tell me about a time you identified an unexpected issue in a manufacturing process and how you resolved it."

 "Tell me about a time you noticed a problem before it became a major issue."

  "Give me an example of how you ensured product quality under challenging circumstances."

  "Tell me about a time you had to take immediate action to minimize production risk."

  "Describe a situation where you used data/statistical analysis to validate a solution."

  "Tell me about a time you implemented preventive measures after solving a problem."

✅ STAR Answer #1: Contamination & Tool Recovery in Ni Bath (Filter Debris Case)

✅ S – Situation

While working as a Process Engineer at Intel, I noticed discoloration around the filter housing on the Ni bath for the CEV503 tool. Upon closer inspection, I found white, fibrous debris accumulating on the filter surface. This contamination posed a risk to product quality and needed urgent resolution.

✅ T – Task

My task was to investigate the root cause, evaluate the impact on production, and execute a recovery plan to bring the tool back online while minimizing production loss and risk to product integrity.

✅ A – Action

- I immediately took the tool offline and contained the POR lots before they exited the factory.

- I evaluated key tool parameters such as pump RPM and bath flow rate using E3 real-time monitoring to assess operational impact — all readings were within acceptable ranges.

- I ran test wafers (blankets and BMPs) through the contaminated bath and sent both wafers and bath samples for lab analysis.

- I followed standard recovery procedures: dumped the bath, performed a superclean with DI and sacrificial flush, and replaced all filters before refilling the bath.

- I conducted statistical analysis on test wafer data (thickness, voids) using OVZ metrology to confirm there were no lingering effects.

- Lastly, I led an After Action Review (AAR), hypothesizing the likely cause to be cleanroom wipe remnants, and proposed multiple corrective measures.

✅ R – Result

The tool was qualified successfully with no yield loss. The AAR led to lasting procedural changes:

- Flashlight inspection added during PMs

- New BC (bunny suit) protocols for open-bath work

- Wipe count tracking

- Enhanced visual signage and communication for work-in-progress
These changes reduced the risk of future contamination events and improved operator awareness.

✅ root-cause analysis, technical problem solving, process optimization, and innovation under constraints.

Question:

"Tell me about a time you solved a recurring quality issue by changing a process parameter or workflow."

"Tell me about a time you solved a recurring quality issue."

"Give me an example of when you optimized a process for efficiency or yield."

"Describe a time when you had to validate a hypothesis with data before making changes."

"Tell me about a time you dealt with equipment limitations and still achieved results."

"Have you ever developed a solution that was later implemented as a standard procedure?"

STAR Answer #2: Stray Current Impact and Cu2 Plating Optimization

✅ S – Situation

In the Cu2 plating process for DoW25 IO CNCS products, we were experiencing frequent delamination at the Ni-Cu2 interface, leading to major defects during etching. Initial analysis showed the issue correlated with specific plating cells and not recipe parameters, suggesting deeper electrical issues.

✅ T – Task

I needed to investigate the source of the defect, validate the root cause, and implement a process-level change to eliminate delamination — while maintaining compatibility with existing tooling and process flows.

✅ A – Action

- I performed a correlation study and found that some Cu2 plating cells introduced non-zero stray current during soak — despite the recipe requesting zero current.

- I validated this theory by measuring the stray current using DVMs and oscilloscopes, and confirmed the defect followed the power supply unit, not the wafer or lot.

- I tested three soak-time options:

  - Immediate PS enable before plating

  - Short 5s soak

  - Hot dwell + 1s soak

- I ran extensive trials using known-bad cells and validated performance with aggressive WCF etch hammers.

- Based on results, I selected the Hot dwell + 1s soak method, which minimized stray current exposure while avoiding bath contamination from anodic depolarization.

✅ R – Result

The updated soak approach eliminated delamination defects without needing hardware or software changes. This method was formally documented in a white paper (Rev2 pilot) and adopted into standard procedures. Our solution restored plating reliability and enabled higher yield on high-value CNCS products.

Examples of collaborations:

- Collab with floor in troubleshooting of tools and ensuring on spec performace, Factory tool down timr schedule

- with integration for lot containment and DRBs,

- With Vendors on tool BKM,

Spec Edits:

For example adding flashlight inspection of plating filters during weekly pMs

Automation: AM edits to add a product and process to AM after NPI- to exclude any product or tool from accepting new

Meeting minutes:

Goals for meeting:

- Monitor the CEV PCSA final height data on a set scheduled frequency in order to catch marginalities in the tools/ duets/ products/ etc. before they start to flag. React to any flags to ensure our tools are healthy.

  - Go through 1 click to find root cause of PCSA flags or at least review the flag and assign owner to resolve issue

  - Review tool health potential issues (Entities/Duets) and assign owner to resolve issue

Resume changes:

Root cause analysis: technical failure investigation: managing one or more technical failure investigation — from identification to conclusion and mitigation / solution development.

Work on assigned Root Cause investigation - coordinate cross functional investigation teams, derive conclusions in alignment with the investigation team and experts. Perform review, create RCA presentation and report. Present investigation status, results to management and customers.